EyeQ DG Diagnostics:

EyeQ DG functionality performing below diagnostic checks , the logs the fault if there is any failure.

- Sync Frame index Check

- Message Timeout Check

- Missing Message check

- Crc Diagnostics check

Chap 1

There are many messages are coming from Mobileye chip and they are categories as  Object,Lane,…, for each message there will be diagnostic check.

For each message, files are created in \Application\Core_Comp\CDD\EyeQ_CDD\EyeQ_DG\src

Consider an example of an Object information for that “EyeQ_DG_OBJT_Process.c”

Whatever Object related signal present in Jupiter/norma sheet each signal is handled here

When application (EyeQWrapper,….)  need any signal information then it is calling function inside(EyeQ_DG_OBJT_Process.c) this.

Suppose application need information regarding OBJ_Inv_TTC_V  then it is calling to function EYEQDG_Get_OBJT_OBJ_Inv_TTC_V  ,it send address of application variable

Then this application variable data filled with core application buffer variable “EYEQDG_OBJT_ParamsApp_s” as shown in below diagram

But who will fill this application core buffer “EYEQDG_OBJT_ParamsApp_s” we will see further.

Chap 2

Application core buffer “EYEQDG_OBJT_ParamsApp_s” which is using by application SWC component will filled by EYEQDG_OBJT_Params_s

In above image

- Application core buffer filled with Header part

- Application core buffer filled with Object part

- 2nd for loop is for safe check ,is that all data filled

By seeing above code it is understand that initially data get into core buffer EYEQDG_OBJT_Params_s and then it is giving to application core buffer EYEQDG_OBJT_ParamsApp_s

So how core buffer EYEQDG_OBJT_Params_s will get update this we will check into next chapter.

Chap 3

In IPC_User.c file we are giving the address of Core buffer EYEQDG_OBJT_Params_s ,in this address camera Received data filled

Once the data filled call back function also called, in this case EYEQDG_OBJT_msgRxCallback

This call back function located in “EyeQ_DG__Common_Process.c” file where all message related information updated like

- Updates OBJT message tracker after new message is received

- Update number of messages received

- Confirm that correct number of objects are received

- NOTIFY_MASK

This NOTIFY_MASK  confirms that message is received and this MASK we are checking in eyeq wrapper before process to message. If MASK is OK then we are checking CRC,if it is applicable

If any message related fault, then it is also set in this function.

Chap 4

CRC Check

For those messages CRC applicable is mentioned in “EyeQ_DGServices.c” file

Consider object message

And this CRC function is called  inside function EYEQDG_IsMessageValid

So to process any message in eyeq wrapper we are calling to function EYEQDG_IsMessageValid

to check CRC is valid or not.

If YES then we read signals and

If NO then we set CRC fail and doing further action.

Check below image for the same.

Figure 15 Core and Application buffer interaction

IPC uses core buffers to store Information sent/received from EyeQ. After receiving message from EyeQ IPC will process the message and copies the information to Core buffers. Once all diagnostics passed and data is ready to use by application software, the content of Core buffers will be copied into Application buffers. These Application buffers are used by EyeQ_DG module. The content of Application buffers will be exchnaged through generated DG interfaces with Application software.

- Useful variable during release activity (integration test)

- EYEQP_SystemMainState_u8

#define C_EYEQSYSS_SYSTEM_STARTUP_STATE                  	( 2U )

#define C_EYEQSYSS_SYSTEM_NORMAL_STATE                   	( 3U )

#define C_EYEQSYSS_SYSTEM_TEMP_UNAVAILABLE_STATE        ( 4U )

#define C_EYEQSYSS_SYSTEM_SAFE_STATE                     	( 5U )

#define C_EYEQSYSS_SYSTEM_UNAVAILABLE_STATE              	( 6U )

#define C_EYEQSYSS_SYSTEM_SHUTDOWN_STATE                 	( 7U )

#define C_EYEQSYSS_NUMBER_OF_SYSTEM_STATES               	( 8U )

- EYEQS_CurrentMainState_u8

#define C_EYEQBS_EYEQ_MAIN_STATE_UNKNOWN                       ( 0x00U )

#define C_EYEQBS_EYEQ_MAIN_STATE_PENDING                       	( 0x01U )

#define C_EYEQBS_EYEQ_MAIN_STATE_BOOT                          	( 0x03U )

#define C_EYEQBS_EYEQ_MAIN_STATE_PENDING_DV                    ( 0x85U )

#define C_EYEQBS_EYEQ_MAIN_STATE_RUNNING_APP_BURN      ( 0xAAU )

#define C_EYEQBS_EYEQ_MAIN_STATE_PENDING_VISION              ( 0x92U )

#define C_EYEQBS_EYEQ_MAIN_STATE_VISION                        	 ( 0x02U )

#define C_EYEQBS_EYEQ_MAIN_STATE_PENDING_SPC                   ( 0x82U )

#define C_EYEQBS_EYEQ_MAIN_STATE_RUNNING_SPC                   ( 0x22U )

#define C_EYEQBS_EYEQ_MAIN_STATE_PENDING_SPTAC               ( 0x81U )

#define C_EYEQBS_EYEQ_MAIN_STATE_RUNNING_SPTAC               ( 0x21U )

#define C_EYEQBS_EYEQ_MAIN_STATE_PENDING_TAC                   ( 0x80U )

#define C_EYEQBS_EYEQ_MAIN_STATE_RUNNING_TAC                   ( 0x20U )

#define C_EYEQBS_EYEQ_MAIN_STATE_PENDING_TAC2                  ( 0xABU )

#define C_EYEQBS_EYEQ_MAIN_STATE_RUNNING_TAC2                  ( 0xACU )

#define C_EYEQBS_EYEQ_MAIN_STATE_PENDING_SFR                   ( 0x83U )

#define C_EYEQBS_EYEQ_MAIN_STATE_RUNNING_SFR                   ( 0xB0U )

#define C_EYEQBS_EYEQ_MAIN_STATE_PENDING_SP                    ( 0x89U )

#define C_EYEQBS_EYEQ_MAIN_STATE_RUNNING_SP                    ( 0xAEU )

#define C_EYEQBS_EYEQ_MAIN_STATE_RUNNING_STEREO              ( 0xAFU )

#define C_EYEQBS_EYEQ_STATE_UNAVAILBLE                         	( 0xFFU )

- EyeQ_RestartAttemptCnt_u16

- EyeQLastResetSource_u8

- EYEQS_CurrentSubState_u8

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_CORESYSSTART_INIT              ( 0x00U )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_CORESYSSTART_FAILURE           ( 0x0EU )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_CORESYSSTART_SUCCESS           ( 0x0FU )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_PWRVISION_INIT                 ( 0x10U )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_PWRVISION_FAILURE              ( 0x1EU )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_PWRVISION_SUCCESS              ( 0x1FU )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SBS_INIT                       ( 0x20U )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SBS_HASHCALCULATION            ( 0x21U )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SBS_SIGNATUREVERIFICATION      ( 0x22U )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SBS_FAILURE                    ( 0x2EU )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SBS_SUCCESS                    ( 0x2FU )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SUSD_INIT                      ( 0x30U )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SUSD_BIST                      ( 0x31U )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SUSD_BOOTAUTHENTICATION        ( 0x32U )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SUSD_DDRINIT                   ( 0x33U )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SUSD_APPLICATIONAUTHENTICATION ( 0x34U )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SUSD_APPMODEPENDING            ( 0x35U )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SUSD_DDRTEST1                  ( 0x36U )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SUSD_DDRTEST2                  ( 0x37U )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SUSD_DDRTESTFAILURE            ( 0x38U )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SUSD_DDRTESTSUCCESS            ( 0x39U )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SUSD_WAITINGHOTRESTART         ( 0x3BU )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SUSD_DDRTEST_INIT_FAILURE      ( 0x3CU )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SUSD_FAILURE                   ( 0x3EU )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SUSD_SUCCESS                   ( 0x3FU )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SUSD_SPIINITFAILURE            ( 0x40U )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_EYEQ_SWITCH_SOFT_REBOOT        ( 0x50U )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_EYEQ_SWITCH_HARD_REBOOT        ( 0x51U )/* System Substates During Shutdown */

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SHUTDOWN_FIRSTDDRTEST    ( 1U )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SHUTDOWN_SECONDDDRTEST   ( 2U )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_SHUTDOWN_FINISHED        ( 3U )

- #define C_EYEQSYSS_SYSTEM_SUBSTATE_APPLICATION_TURN_OFF_EYEQ_REQUEST_SUCCESS    ( 4U )

- EYEQP_CurrentErrorStatus_u32

/****************** EyeQ Error code Table Start ***************/

- #define C_EYEQDG_EYEQ_NO_ERRORCODE                     ( 0x00U )

- #define C_EYEQDG_EYEQ_VIDEOMSG_TIMEOUT                 ( 0x01U )

- #define C_EYEQDG_EYEQ_VEH_MSG_TIMEOUT                  ( 0x02U )

- #define C_EYEQDG_APPLICATION_SHUTDOWN_REQUEST          ( 0x03U )

- #define C_EYEQDG_SWITCH_APPLICATION_MODE_TRANSITION    ( 0x04U )

- #define C_EYEQDG_APPLICATION_TURN_OFF_EYEQ_REQUEST     ( 0x05U )

/************************ Free slots **************************/

- #define C_EYEQDG_EYEQ_CARSNS_SIGNAL_INTEGRITY_ERR      ( 0x07U )

/************************ Free slots **************************/

- #define C_EYEQDG_EYEQ_CMN_CRC_ERROR                    ( 0x09U )

- #define C_EYEQDG_EYEQ_CAM_INIT_ERR                     ( 0x10U )

- #define C_EYEQDG_EYEQ_I2C_TIMEOUT_ERR                  ( 0x11U )

- #define C_EYEQDG_EYEQ_APPDIAG_VERIFIERS_ERROR          ( 0x12U )

- #define C_EYEQDG_EYEQ_FFS_INTGT_FAILURE                ( 0x13U )

- #define C_EYEQDG_EYEQ_DDR_CHIP_FAILURE                 ( 0x14U )

- #define C_EYEQDG_EYEQ_FLASH_MEM_FAILURE                ( 0x15U )

- #define C_EYEQDG_EYEQ_APP_ERR_FAULT                    ( 0x16U )

- #define C_EYEQDG_EYEQ_APP_INIT_ERR_FAULT               ( 0x17U )

- #define C_EYEQDG_EYEQ_BIST_FAILURE                     ( 0x18U )

- #define C_EYEQDG_EYEQ_TFLSP_VERSION_ERROR              ( 0x19U )

- #define C_EYEQDG_EYEQ_CAL_ERROR                        ( 0x1AU )

- #define C_EYEQDG_EYEQ_CAM_SELF_RESET                   ( 0x1BU )

- #define C_EYEQDG_EYEQ_IPC_SET_INIT_CAL_ERR             ( 0x1CU )

- #define C_EYEQDG_EYEQ_IPC_DTP_ERR                      ( 0x1DU )

- #define C_EYEQDG_EYEQ_IPC_DEAD                         ( 0x1EU )

- #define C_EYEQDG_EYEQ_SYS_VERIFI_REQ_ERROR             ( 0x1FU )

- #define C_EYEQDG_EYEQ_SYS_VERIFI_REQ_TRW_ERROR         ( 0x20U )

- #define C_EYEQDG_EYEQ_MESSAGE_SIZE_MISMATCH            ( 0x21U )

- #define C_EYEQDG_EYEQ_CMN_VERSION_ERROR                ( 0x22U )

- #define C_EYEQDG_EYEQ_TFS_VERSION_ERROR                ( 0x23U )

- #define C_EYEQDG_EYEQ_SAFETY_MSG_CRC_ERROR             ( 0x24U )

- #define C_EYEQDG_EYEQ_FCFVD_VERSION_ERROR              ( 0x25U )

- #define C_EYEQDG_EYEQ_FCFVRU_VERSION_ERROR             ( 0x26U )

- #define C_EYEQDG_EYEQ_CONAR_VERSION_ERROR              ( 0x27U )

- #define C_EYEQDG_EYEQ_I2C_VIDEO_GRAB_FAILED            ( 0x28U )

- #define C_EYEQDG_EYEQ_PLL_FAILURE                      ( 0x29U )

- #define C_EYEQDG_EYEQ_PARITY_FAILURE                   ( 0x2AU )

- #define C_EYEQDG_EYEQ_DFA_VERSION_ERROR                ( 0x2BU )

- #define C_EYEQDG_EYEQ_FFS_CORRUPTED_ERR                ( 0x2CU )

- #define C_EYEQDG_EYEQ_TRW_SAFETY_CODE_CRC              ( 0x2DU )

- #define C_EYEQDG_EYEQ_TRW_SAFETY_IMG_GRAB_MISS         ( 0x2EU )

- #define C_EYEQDG_EYEQ_TRW_SAFETY_FCV_SIG_FFI           ( 0x2FU )

- #define C_EYEQDG_EYEQ_TRW_SAFETY_AEB_PARAM_FFI         ( 0x30U )

- #define C_EYEQDG_EYEQ_INCREMENT_SYNC_FRAME_ERROR       ( 0x31U )

- #define C_EYEQDG_EYEQ_PV_GENERAL_FAULT                 ( 0x32U )

- #define C_EYEQDG_EYEQ_PV_VERIFICATION_FAULT            ( 0x33U )

- #define C_EYEQDG_EYEQ_PLL_COMPARISON_FAULT             ( 0x34U )

- #define C_EYEQDG_EYEQ_FLSF_VERSION_ERROR               ( 0x35U )

- #define C_EYEQDG_EYEQ_HLB_VERSION_ERROR                ( 0x36U )

- #define C_EYEQDG_EYEQ_LDM_VERSION_ERROR                ( 0x37U )

- #define C_EYEQDG_EYEQ_LDW_VERSION_ERROR                ( 0x38U )

- #define C_EYEQDG_EYEQ_LNADJ_VERSION_ERROR              ( 0x39U )

- #define C_EYEQDG_EYEQ_LNAPP_VERSION_ERROR              ( 0x3AU )

- #define C_EYEQDG_EYEQ_LNHST_VERSION_ERROR              ( 0x3BU )

- #define C_IPCDRV_SPI_PARTIAL_BYTE_FAILURE              ( 0x3CU )

- #define C_EYEQDG_EYEQ_LSCRF_VERSION_ERROR              ( 0x3DU )

- #define C_IPCDRV_SPI_BUFFER_MISALIGNMENT_ERROR         ( 0x3EU )

- #define C_EYEQDG_EYEQ_MISSING_MSG_ERROR                ( 0x3FU )

- #define C_EYEQDG_EYEQ_LSCST_VERSION_ERROR              ( 0x40U )

- #define C_EYEQDG_EYEQ_LSCVC_VERSION_ERROR              ( 0x41U )

- #define C_EYEQDG_EYEQ_CAM_PARAMS_CCFT_CRC_FAILED       ( 0x42U )

- #define C_EYEQDG_EYEQ_APP_GVPU_STATE_TERMINA           ( 0x43U )

- #define C_EYEQDG_EYEQ_APPLMSG_VERSION_ERROR            ( 0x44U )

- #define C_EYEQDG_EYEQ_OBJT_VERSION_ERROR               ( 0x45U )

- #define C_EYEQDG_EYEQ_MSG_TIMEOUT_INDICATION           ( 0x46U )

- #define C_EYEQDG_EYEQ_MODE_TRANSITION_ERROR            ( 0x47U )

- #define C_EYEQDG_BOARD_REVISION_MISMATCH_ERROR         ( 0x48U )

- #define C_EYEQDG_EYEQ_LOFC_VERSION_ERROR               ( 0x49U )

- #define C_EYEQDG_SAFETY_MSG_INPUT_CRC_MISMATCH         ( 0x4AU )

- #define C_EYEQDG_EYEQ_RPFL_VERSION_ERROR               ( 0x4BU )

- #define C_EYEQDG_EYEQ_RSD_VERSION_ERROR                ( 0x4CU )

- #define C_EYEQDG_EYEQ_SMNLIN_VERSION_ERROR             ( 0x4DU )

- #define C_EYEQDG_BOOT_UNSUPPORTED_EYEQ_VERSION         ( 0x4EU )

- #define C_EYEQDG_EYEQ_SMNLND_VERSION_ERROR             ( 0x4FU )

- #define C_EYEQDG_EYEQ_THERMAL_SHUTDOWN                 ( 0x50U )

- #define C_EYEQDG_EYEQ_GPIO16_EYEQ_WARMUP_STATE         ( 0x51U )

- #define C_EYEQDG_EYEQ_GPIO16_EYEQ_CORE_DUMP            ( 0x52U )

- #define C_EYEQDG_EYEQ_SMNMRK_VERSION_ERROR             ( 0x53U )

- #define C_EYEQDG_EYEQ_MCD_CORE_DUMP_END                ( 0x54U )

- #define C_EYEQDG_EYEQ_MCD_DATA_READ_ERR                ( 0x55U )

- #define C_EYEQDG_EYEQ_SDIAG_VERSION_ERROR              ( 0x56U )

- #define C_EYEQDG_EYEQ_FSP_VERSION_ERROR                ( 0x57U )

- #define C_EYEQDG_EYEQ_HRVC_VERSION_ERROR               ( 0x58U )

- #define C_EYEQDG_EYEQ_IPC_ME_COMPATIBILITY             ( 0x59U )

- #define C_EYEQDG_EYEQ_IPC_ME_RESP_TIMEOUT              ( 0x60U )

- #define C_EYEQDG_EYEQ_IPC_RX_FCS_FAILURE               ( 0x61U )

- #define C_EYEQDG_EYEQ_IPC_RX_UNREG_PROT                ( 0x62U )

- #define C_EYEQDG_EYEQ_IPC_RX_UNREG_ID                  ( 0x63U )

- #define C_EYEQDG_EYEQ_IPC_RX_FIRST_FRAME_ERR           ( 0x64U )

- #define C_EYEQDG_EYEQ_IPC_RX_RX_OVERRUN                ( 0x65U )

- #define C_EYEQDG_EYEQ_IPC_RX_FRAME_SEQ_ERROR           ( 0x66U )

- #define C_EYEQDG_EYEQ_IPC_APPL_BUFF_OVF                ( 0x67U )

- #define C_EYEQDG_EYEQ_IPC_GEN_ERR                      ( 0x68U )

- #define C_EYEQDG_EYEQ_IPC_DRV_BUFF_OVF                 ( 0x69U )

- #define C_EYEQDG_EYEQ_HZD_VERSION_ERROR                ( 0x6AU )

- #define C_EYEQDG_EYEQ_LNRE_VERSION_ERROR               ( 0x6BU )

- #define C_EYEQDG_EYEQ_LOHA_VERSION_ERROR               ( 0x6CU )

- #define C_EYEQDG_EYEQ_TFLST_VERSION_ERROR              ( 0x6DU )

- #define C_EYEQDG_EYEQ_POWER_VBAT_ERROR                 ( 0x6EU )

- #define C_EYEQDG_EYEQ_POWER_1V0_ERROR                  ( 0x70U )

- #define C_EYEQDG_EYEQ_POWER_1V1_ERROR                  ( 0x71U )

- #define C_EYEQDG_EYEQ_POWER_1V5_ERROR                  ( 0x72U )

- #define C_EYEQDG_EYEQ_POWER_1V8_ERROR                  ( 0x73U )

- #define C_EYEQDG_EYEQ_POWER_1V3_ERROR                  ( 0X74U )

- #define C_EYEQDG_EYEQ_POWER_3V3_ERROR                  ( 0x75U )

- #define C_EYEQDG_EYEQ_POWER_5V0_ERROR                  ( 0x76U )

- #define C_EYEQDG_EYEQ_POWER_VBAT_INIT_ERROR            ( 0x77U )

- #define C_EYEQDG_EYEQ_POWER_3V3_INIT_ERROR             ( 0x78U )

- #define C_EYEQDG_EYEQ_POWER_5V0_INIT_ERROR             ( 0x79U )

- #define C_EYEQDG_EYEQ_POWER_2V8_ERROR                  ( 0x7AU )

- #define C_EYEQDG_EYEQ_POWER_1V2_ERROR                  ( 0X7BU )

- #define C_EYEQDG_EYEQ_POWER_1V0_OFFLOAD_ERROR          ( 0X7CU )

- #define C_EYEQDG_EYEQ_POWER_1V1_OFFLOAD_ERROR          ( 0X7DU )

- #define C_EYEQDG_EYEQ_POWER_IMAGER_OFFLOAD_ERROR       ( 0X7EU )

- #define C_EYEQDG_EYEQ_POWER_1V8_OFFLOAD_ERROR          ( 0X7FU )

- #define C_EYEQDG_EYEQ_SFR_VERSION_ERROR                ( 0x80U )

- #define C_EYEQDG_EYEQ_DYN_CALIB_VERSION_ERROR          ( 0x81U )

- #define C_EYEQDG_EYEQ_SP_VERSION_ERROR                 ( 0x82U )

- #define C_EYEQDG_EYEQ_TTPE_VERSION_ERROR               ( 0x83U )

- #define C_EYEQDG_EYEQ_SENS_MSG_TIMEOUT                 ( 0x84U )

- #define C_EYEQDG_CALSTCMSG_MODE_ERROR                  ( 0x85U )

- #define C_EYEQDG_CALSTCMSG_HDRCRC_ERROR                ( 0x86U )

- #define C_EYEQDG_CALSTCMSG_OBJCRC_ERROR                ( 0x87U )

- #define C_EYEQDG_CALSTCMSG_VERSION_ERROR               ( 0x88U )

- #define C_EYEQDG_EYEQ_BOOT_VERSION_ERROR               ( 0x89U )

- #define C_EYEQDG_EYEQ_TTPM_VERSION_ERROR               ( 0x8AU )

- #define C_EYEQDG_EYEQ_EDR_NOT_ESPRUNNING_ONTIME        ( 0x8BU )

- #define C_EYEQDG_VEHICLE_SIGS_INVALID                  ( 0x8CU )

- #define C_EYEQDG_NVM_ERROR                             ( 0x90U )

- #define C_EYEQDG_EYEQ_DYN_CALIB_CRC_HEADER_ERROR       ( 0x91U )

- #define C_EYEQDG_EYEQ_DYN_CALIB_CRC_OBJ_ERROR          ( 0x92U )

- #define C_EYEQDG_EYEQ_DYN_CALIB_WRONG_MODE_ERROR       ( 0x93U )

- #define C_EYEQDG_EYEQ_IPC_RX_FRAME_HDR_ERROR           ( 0x94U )

- #define C_EYEQDG_EYEQ_IPC_RX_FRAME_CRC_ERROR           ( 0x95U )

- #define C_EYEQDG_EYEQ_IPC_CONS_FRAME_NOT_RXED          ( 0x96U )

- #define C_EYEQDG_EYEQ_IPC_RX_IMPROPER_FRAME_SIZE       ( 0x97U )

- #define C_EYEQDG_EYEQ_IPC_INCOMPLETE_MULTFRM_ERR       ( 0x98U )

- #define C_EYEQDG_FLSF_HDRCRC_ERROR                     ( 0X99U )

- #define C_EYEQDG_FLSF_OBJCRC_ERROR                     ( 0X9AU )

- #define C_EYEQDG_EYEQ_TTPT_VERSION_ERROR               ( 0x9BU )

- #define C_EYEQDG_EYEQ_TTPTD_VERSION_ERROR              ( 0x9CU )

- #define C_EYEQDG_EYEQ_CAM_INIT_CAMERA_EEPROM           ( 0x9DU )

- #define C_EYEQDG_EYEQ_GRAB_CAMERA_FAILED               ( 0x9EU )

- #define C_EYEQDG_EYEQ_CPS_STL_FAIL                     ( 0x9FU )

- #define C_EYEQDG_EYEQ_DDR_DRIFT_COMPARISON_FAIL        ( 0xA0U )

- #define C_EYEQDG_EYEQ_EDR_FLASH_FAIL                   ( 0xA1U )

/******************* Safety errors zone - Start ****************/

- #define C_EYEQDG_FS_OOC_ERROR                          ( 0xA2U )

- #define C_EYEQDG_EYEQ_INTERMITTENT_CNR_ERROR           ( 0xA3U )

- #define C_EYEQDG_SAFETY_BIT_SET                        ( 0xA4U )

- #define C_EYEQDG_EYEQ_MISSING_FIRST_CNR_ERROR          ( 0xA5U )

- #define C_EYEQDG_EYEQ_CHALLENGE_ERROR                  ( 0xA6U )

- #define C_EYEQDG_EYEQ_WRONG_CNR_ERROR                  ( 0xA7U )

- #define C_EYEQDS_FS_MISALIGN_YAW_TIMEOUT_ERROR         ( 0xA8U )

- #define C_EYEQDS_FS_MISALIGN_PITCH_TIMEOUT_ERROR       ( 0xA9U )

- #define C_EYEQDS_FS_OOF_ERROR                          ( 0xAAU )

- #define C_EYEQDG_SAFETY_LD_IND_SET                     ( 0xABU )

- #define C_EYEQDG_SAFETY_VM_IND_SET                     ( 0xACU )

- #define C_EYEQDG_SAFETY_PED_IND_SET                    ( 0xADU )

- #define C_EYEQDG_SAFETY_BIT30_SET                      ( 0xAEU )

/************************ Free slots **************************/

- #define C_EYEQDG_SAFETY_BIT36_SET                      ( 0xB0U )

- #define C_EYEQDG_SAFETY_BIT39_SET                      ( 0xB1U )

- #define C_EYEQDG_SAFETY_BIT42_SET                      ( 0xB2U )

- #define C_EYEQDG_SAFETY_BIT43_SET                      ( 0xB3U )

- #define C_EYEQDG_SAFETY_BIT45_SET                      ( 0xB4U )

- #define C_EYEQDG_SAFETY_BIT48_SET                      ( 0xB5U )

- #define C_EYEQDG_SAFETY_BIT51_SET                      ( 0xB6U )

- #define C_EYEQDG_SAFETY_BIT52_SET                      ( 0xB7U )

- #define C_EYEQDG_SAFETY_BIT4_SET                       ( 0xB8U )

- #define C_EYEQDG_SAFETY_BIT6_SET                       ( 0xB9U )

- #define C_EYEQDG_SAFETY_BIT7_SET                       ( 0xBAU )

- #define C_EYEQDG_SAFETY_BIT9_SET                       ( 0xBBU )

- #define C_EYEQDG_SAFETY_BIT18_SET                      ( 0xBCU )

- #define C_EYEQDG_SAFETY_BIT24_SET                      ( 0xBDU )

- #define C_EYEQDG_SAFETY_BIT25_SET                      ( 0xBEU )

- #define C_EYEQDG_SAFETY_BIT26_SET                      ( 0xBFU )

- #define C_EYEQDG_SAFETY_BIT27_SET                      ( 0xC0U )

- #define C_EYEQDG_SAFETY_BIT31_SET                      ( 0xC1U )

- #define C_EYEQDG_SAFETY_BIT34_SET                      ( 0xC2U )

- #define C_EYEQDG_SAFETY_BIT35_SET                      ( 0xC3U )

- #define C_EYEQDG_SAFETY_BIT37_SET                      ( 0xC4U )

- #define C_EYEQDG_SAFETY_BIT38_SET                      ( 0xC5U )

- #define C_EYEQDG_SAFETY_BIT40_SET                      ( 0xC6U )

- #define C_EYEQDG_SAFETY_BIT44_SET                      ( 0xC7U )

- #define C_EYEQDG_SAFETY_BIT47_SET                      ( 0xC8U )

- #define C_EYEQDG_SAFETY_BIT49_SET                      ( 0xC9U )

- #define C_EYEQDG_SAFETY_BIT50_SET                      ( 0xCAU )

- #define C_EYEQDG_SAFETY_BIT53_SET                      ( 0xCBU )

- #define C_EYEQDG_SAFETY_BIT54_SET                      ( 0xCCU )

- #define C_EYEQDG_SAFETY_BIT11_SET                      ( 0xCDU )

- #define C_EYEQDG_SAFETY_BIT13_SET                      ( 0xCEU )

- #define C_EYEQDG_SAFETY_BIT21_SET                      ( 0xCFU )

- #define C_EYEQDG_SAFETY_BIT29_SET                      ( 0xD0U )

- #define C_EYEQDG_SAFETY_LD_EDGE_IND_SET                ( 0xD1U )

- #define C_EYEQDG_SAFETY_AEB_IND_SET                    ( 0xD2U )

/*************** Safety errors zone - End **********************/

- #define C_EYEQDG_EYEQ_MSG_TIMEOUT                      ( 0xD3U )

- #define C_EYEQDG_EYEQ_INVALID_REGION_CODE              ( 0xD4U )

- #define C_EYEQDG_INVALID_VEHICLE_SIGNALS_ERROR         ( 0xD5U )

- #define C_IOHWAB_PWMGR_BOARDREV_INVALID_ERROR          ( 0xD6U )

/*****************SBS error****************************/

- #define C_EYEQ_SBS_AUTHENTICATION_FAILED_ERROR         ( 0xD7U )

/*****************ADC test error****************************/

- #define C_IOHWAB_ADCDIAG_PULLDOWN_DIAG_ERROR           ( 0xD8U )

- #define C_IOHWAB_ADCDIAG_CONVERTER_ERROR               ( 0xD9U )

- #define C_IOHWAB_ADCDIAG_BROKEN_WIRE_ERROR             ( 0xDAU )

/*****************ErrorOutPin error****************************/

- #define C_IOHWAB_DIODIAG_FATAL_ERROR                   ( 0xDBU )

- #define C_IOHWAB_DIODIAG_SHORT2GND_ERROR               ( 0xDCU )

- #define C_IOHWAB_DIODIAG_SHORT2HIGH_ERROR              ( 0xDDU )

- #define C_IOHWAB_DIODIAG_OVERTEMP_ERROR                ( 0xDEU )

/*******************Thermistor Sensor Failure *****************/

- #define C_IOHWAB_THERMISTOR_SENSOR_FAILURE             ( 0XDFU )/*Aurix 2G Thermistor Sensor Failure*/

/************** Startup/Shutdown Error Zone - Start ***********/

- #define C_EYEQDG_BOOT_TIMEOUT_BIST                     ( 0xE0U )

- #define C_EYEQDG_BOOT_TIMEOUT_BOOTAUTH                 ( 0xE1U )

- #define C_EYEQDG_BOOT_TIMEOUT_RAMINIT                  ( 0xE2U )

- #define C_EYEQDG_BOOT_TIMEOUT_APPAUTH                  ( 0xE3U )

- #define C_EYEQDG_BOOT_TIMEOUT_DDRTEST                  ( 0xE4U )

- #define C_EYEQDG_BOOT_SEQUENCE_ERROR_BIST              ( 0xE5U )

- #define C_EYEQDG_BOOT_SEQUENCE_ERROR_BOOTAUTH          ( 0xE6U )

- #define C_EYEQDG_BOOT_SEQUENCE_ERROR_RAMINIT           ( 0xE7U )

- #define C_EYEQDG_BOOT_SEQUENCE_ERROR_APPAUTH           ( 0xE8U )

- #define C_EYEQDG_BOOT_SEQUENCE_ERROR_DDRTEST           ( 0xE9U )

- #define C_EYEQDG_BOOT_DDR_TEST_FIRST_FAILURE           ( 0xEAU )

- #define C_EYEQDG_BOOT_DDR_TEST_FAILURE                 ( 0xEBU )

/************************ Free Space **************************/

- #define C_EYEQDG_BOOT_BOOT_AUTH_FAILURE                ( 0xEDU )

- #define C_EYEQDG_BOOT_APP_AUTH_FAILURE                 ( 0xEEU )

- #define C_EYEQDG_BOOT_DDRTEST_INIT_FAILURE             ( 0xEFU )/*DDR Initialization Failure*/

/************ Startup/Shutdown Error Zone - end ***************/

- #define C_EYEQNVM_CORE_NVM_BLOCK_CRC_FAILED            ( 0XF0U )

- #define C_EYEQNVM_CORE_NVM_BLOCK_TAGID_FAILED          ( 0XF1U )

- #define C_IOHWABNVM_CORE_NVM_BLOCK_CRC_FAILED          ( 0XF2U )

- #define C_IOHWABNVM_CORE_NVM_BLOCK_TAGID_FAILED        ( 0XF3U )

- #define C_IOHWAB_AURIX_DIE_SENSOR_FAILURE              ( 0XF4U )/*Aurix 2G Die Sensor Failure*/

- #define C_IOHWAB_GPIO18_VERIFICATION_ERROR             ( 0xF5U )

- #define C_IOHWAB_CYPRESS_FLASH_FAIL                    ( 0xF6U )

- #define C_EYEQDG_EYEQ_IPC_QSPI0_ERROR                  ( 0xF7U )/*IPCDRV QSPI Errors*/

- #define C_EYEQDG_EYEQ_IPC_QSPI1_ERROR                  ( 0xF8U )

- #define C_EYEQDG_EYEQ_IPC_QSPI2_ERROR                  ( 0xF9U )